Check if the Wiring is Correct

Start of Speed Mode Commissioning

Pr0.00= Motor power

Restore Factory Parameters (Pr5.01=1)

Pr0.01= Motor voltage

Pr0.02= Motor current

Pr0.03= Motor frequency

Input Motor Nameplate Parameters

Pr0.04= Motor speed

Motor parameter input completed

Pr0.26=1  Static tuning

Pr0.26=1  静态调谐

Troubleshoot according to fault codes; contact the manufacturer if unresolved

If a fault is reported

Press the Confirm Key; the Operation Panel Displays ARUN and Flashes. Then Press the RUN Key; the Drive Enters Tuning State. (After Tuning is Completed, the Drive Operation Panel Automatically Returns to Parameter Display State)

Tuning steps

调谐步骤

Start motor tuning

Tuning completed

调谐完成

Modify Parameter Pr0.18=0/1 or After Power - off, Swap Any Two Motor Wires, then Power on and Retune (Retuning is Mandatory; Direct Operation is Not Allowed)

If the rotation direction is incorrect

Method

方法

Check if the motor rotation direction is correct

Set Pr0.12=5 (Set Frequency), Press the Run Key, Check if the Motor Rotation Direction Matches the Oil Pump Direction

Rotation direction correct

1. Modify Command Source Parameter Pr9.06=1, Select Command Source (Terminal Command)
2. Modify Main Frequency Source Parameter Pr9.07=2 (Corresponding to AI1), (3 Corresponding to AI2), (4 Corresponding to AI3)